W2160 User Datasheet

(V1.0)

Beijing Watchdata Co. Ltd.

CONTENTS

1.	GENERAL DESCRIPTION	3

2.	READER FEATURES	3

3.	COMMUNICATION PROTOCOL OF READER	5

4.	THE FUNTIONS OF DLL	7

5.	SELF-DEFINED COMMAND	10

# GENERAL DESCRIPTION

W2160 is a highly integrated USB card reader, it integrates image scanning technology on the traditional smart card reader .It can support  QR code or bar code scanning , and has a contact card slot and support contactless card operation.  The reader also has a magnetic stripe card reading function, and can be equipped with a second-generation ID card information reading function to meet various needs of customers.

# READER FEATURES

  - Support the contact card that satisfies standard ISO 7816－1/2/3/4

  - Support the contact I2C memory card, SLE4428, SLE4442,AT88SC1608

  - Support the contactless card that satisfies standard ISO14443 TYPEA and TYPEB

  - Support the Magnetic Strip card satisfies standard ISO 7810, ISO 7811,ISO 7812 and ISO 7813

  - full speed USB communication ,HID protocol

  - LED indicates the power or communication status

  - Embedded buzzer that can be controlled by the user

  - With the DLL, user can operate card conveniently

| Module | Description |
| --- | --- |
| Scanner | 2D:QR code/ PDF417code; 1D: Code 128, Code 93, F-6, ITF-14, EAN-13, MSI- Plessey, Code 39 ,Code 11,etc. |
| Support Card type | Contact card : ID-1 size slot;Support T0, T1 protocol,I2C Memory Card ,SLE4428,SLE4442, AT88SC 1608; Support 1.8V/3V/5V card Contactless card： TYPEA /TYPEB/MAFIRE ONE Read Range of Contactless Card is 0～4.5CM Magnetic strip card: Support the second track and the third track reading PSAM: Support 4 PSAM slots |
| Communication | Full Speed USB Interface with 12Mbps,HID communication Protocol |
| Indication | 3 LEDS & Buzzer; the red led indicates power status, the green led indicates contact card operation status, the blue led indicates contactless card operation status |
| Power Supply | USB power supply |
| MTBF | 5000 hours |
| Working Current | ≤500mA |
| Dimension | 127*79*65(mm) |
| Temperature | 0℃ ～ 50℃ |
| Humidity | 20% ～ 90% |
| Shell Material | ABS |
| Operation system | Win XP/ Vista/ Win 7/Win 8/ Win 10/linux/ |

# COMMUNICATION PROTOCOL OF READER

Remarks：All the command data is in hex .

PC –to-Reader data

| Protocol data | Abbreviations | lenth | Data Definitions |
| --- | --- | --- | --- |
| Headers | NAD （slot ID） | 1 | 0x12：contact user card 0x15：contactless card 0x16：SAM slot1 0x17：SAM slot2 0x18：SAM slot3 0x19：SAM slot4 0x1A：Magnetic Strip card 0x00：self-defined command to reader |
|  | LEN1 | 1 | High byte of the APDU length |
|  | LEN0 | 1 | Low byte of the APDU length |
| APDU | CLA | 1 |  |
|  | INS | 1 |  |
|  | P1 | 1 |  |
|  | P2 | 1 |  |
|  | Lc | 1 |  |
|  | DATA | 0-FF |  |

- (DLL function can add the headers automatically ,the user only need organize the data)

Reader-to-PC data

| Protocol data | Abbreviations | lenth | Data Definitions |
| --- | --- | --- | --- |
| Headers | NAD | 1 | Slot ID exchange its half-byte |
|  | LEN1 | 1 | 0x00 |
|  | LEN0 | 1 | Low byte of the data and SW |
| Data | DATA | 0-FF | Card`s data |
|  | SW1 | 1 | Status word high byte |
|  | SW2 | 1 | Status word low byte |

- (DLL function can add the headers automatically ,the user only need organize the data)

- Example1：Reset  Contact CPU card

Reader to PC

12    00    05    00    12    00    00    00

↓     ↓      ↓      ↓   ↓       ↓    ↓     ↓

NAD  LEN1  LEN0  CLA  INS    P1   P2     Lc

Reader to PC

- 21   00   13   3B6D0000574446224A864341301F131C12   90   00

- ↓    ↓      ↓              ↓                          ↓   ↓

- NAD  LEN1  LEN0  (←-―――――D A T A－―-―――→)  SW1  SW2

- Example2：Select CPU card MF（3F00）

- PC to Reader:

- 12    00    07    00    A4    00    00    02    3F    00

- ↓      ↓     ↓     ↓     ↓     ↓     ↓     ↓     ↓   ↓   ↓

- NAD  LEN1  LEN0  CLA  INS    P1   P2     Lc     (D A T A)

Reader to PC

- 21   00    02     61   XX

- ↓     ↓     ↓       ↓   ↓

- NAD  LEN1  LEN0  SW1  SW2

# THE FUNTIONS OF DLL

### １）  open the device function

HANDLE WINAPI CT_open(

char *name,

unsigned int param1,

unsigned char param2

);

Parameters：

name:	　"hid1","hid2","hidN" etc, 1<=N<=9

param1:  Default 0

param2:  Default 0

Return ：

INVALID_HANDLE_VALUE(-1)  open failed；

INVALID_HANDLE_VALUE(fd)  open success；

(fd : descriptor of the device)

### 2）Set NAD function

void WINAPI ICC_set_NAD(

HANDLE fd,

unsigned char nad

);

Parameters：

fd	：

nad	：	slot ID (default 0x00)

Return：

NO

………

Example：

ICC_set_NAD(fDev,0x12);	//set NAD is 0x12,contact card slot

### ３）Reset the card

unsigned WINAPI ICC_reset(

HANDLE fd,

unsigned char *lenr,

unsigned char *resp

);

Parameters：

fd	：

lenr	：	the length of ATR (answer to reset )

resp	：	ATR

Return status：

0x9000　 Success

0x6200　 No Ccard

0x6FF0　 Time Out

### ４）send APDU function

Comm. ：CLA INS P1 P2 Lc DATA [Le]

Resp   ：DATA

unsigned WINAPI ICC_tsi_api(

HANDLE fd,

unsigned char len,

unsigned char *comm,

unsigned char *lenr,

unsigned char *resp

);

Parameters：

fd	  ：

len	  ：The Length of comm

comm. ：APDU

lenr	  ：The length of the data from card

resp	  ：The data from card

Return status：

0x6FF0: Time Out

Others: SW1 SW2 from card

### 5）close the device function

int WINAPI CT_close(

HANDLE fd

);

Parameter：

fd	：the return of  CT_open

Return:

-1：failed

0：success

Operating Process of the card

CT_open			//open the device

ICC_set_NAD		//Set slot ID (default 0x00)

ICC_reset		//Reset card

…

ICC_tsi_api		//send APDU

...

CT_close		    //Close the device

Application Example:

Get The Version Of  Reader：

HANDLE fDev;

unsigned char lens;		//

unsigned char lenr;		//

unsigned char comm[300];	//

unsigned char resp[300];		//

unsigned int sw;			//

char tmpbuf[300];		//

CString strDisplay;		//

fDev =CT_open(hid1,0,0);

ICC_set_NAD(fDev,0x00);	//set NAD  0x00

memcpy(comm,"\x00\x19\x00\x00\x00",5);		// get Version

lens=5;				//

sw=ICC_tsi_api(fDev,lens,comm,&lenr,resp);		//

if(sw!=0x9000)		//

{

MessageBox("Get Reader Firmware Version Error!");

}else

{

BinToCHex((unsigned char *)tmpbuf,resp,lenr);		//

tmpbuf[lenr*2]=0;

strDisplay=CString(tmpbuf);

MessageBox("Firmware Version is "+strDisplay);

}

ICC_set_NAD(fDev,0x12);	//set NAD  0x12 contact card slot

sw=ICC_reset(fDev,&lenr,resp) //reset card

memcpy(comm,"\x00\xa4\x00\x00\x02\x3f\x00",7);		// select MF

lens=7;				//

sw=ICC_tsi_api(fDev,lens,comm,&lenr,resp);		//

# SELF-DEFINED COMMAND

| Self-defined command to device |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NAD | CLA | INS | P1 | P2 | Lc | Data | Specification |
| 00 | 00 | 19 | 00 | 00 | 00 |  | Get The Version Of Reader |
| 00 | 00 | 19 | 01 | 01 | Le | data | Get The Serial Number Of Reader (Le<=20) Data: The Serial Number Of Reader (in hex) |
| 15 | B0 | 15 | 0E | 00 | 01 | 00 | Close RF field |

| Self-defined command to contact card |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NAD | CLA | INS | P1 | P2 | Lc | Data | Specification |
| 12 | 00 | 12 | 00 | 00 | 00 |  | Reset |
|  | 00 | B0 | P1 | P2 | LEN |  | Read data from the memory card SLE4442: P1 default 0；P2 address SLE4428: P1(high byte of address) P2(low byte of address） LEN（data length） LEN=0, read 256 bytes |
|  | 00 | 20 | 00 | 00 | LEN | PIN | Check PIN SLE4442: pin is 3bytes； SLE4428: pin is 2bytes； |
|  | 00 | D6 | P1 | P2 | LEN | Data | Write data to the memory card |
|  | 00 | 24 | 00 | 00 | LEN |  | Change PIN |

| Self-defined command to contactless card |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NAD | CLA | INS | P1 | P2 | Lc | Data | Specification |
| 15 | 00 | 12 | 00 | 00 | 00 |  | Reset CPU card ( ISO14443 TYPEA/B) |
|  | 80 | 11 | 09 | 00 | 00 |  | Reset Mifare one |
|  | 80 | 11 | 02 | 00 | 08 | Key type(1 byte)+ block number(1 byte) +key Key type-keyA:0X60 Key type-keyB:0X61 | Authentication block |
|  | 80 | 11 | 03 | 00 | 01 | block number | Read data |
|  | 80 | 11 | 04 | 00 | 11 | block number+ data(16 bytes) | Write data |
|  | 80 | 11 | 05 | 00 | 05 | block number+ amount (4 bytes) | Init wallet |
|  | 80 | 11 | 06 | 00 | 05 | block number+ amount (4 bytes) | Increase |
|  | 80 | 11 | 07 | 00 | 05 | block number+ amount (4 bytes) | Decrease |

| Self-defined Command to magnetic strip card |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NAD | CLA | INS | P1 | P2 | Lc | Data | Specification |
| 1A | 80 | 17 | 03 | P2 | 00 |  | In KBW mode ,the data of magnetic strip card P2： 2. only the second strip data 3. only the third strip data 6. the second and the third strip data |
|  | 80 | 17 | 03 | 00 | 01 | data | Read magnetic strip data in command mode data： 2. only the second strip data 3. only the third strip data 6. the second and the third strip data |
|  | 00 | 19 | 06 | P2 | 00 |  | Change mode P2： 0，set to KBW mode - set to command mode |
|  | 00 | 19 | 20 | P2 | 00 |  | Waiting time (in command mode) P2: time (S) (time < 180s) |

SW1 SW2：

0x9000：Success

0x6D00：Not  support  command

0x6700：Length error

0x6FF0：Time out (communicate error)

0x6985：Write-protect (memory  card )

0x6982： Not reach the secure state ( mifare one )

0x63CX：X is the number of attempt ( mifare one )

0x 6983/6984：Authentication failed ( mifare one )

0x 6FF0：Init error ( mifare one )

0x 6FF3：Write error (mifare one)

0x6902：Time out (magnetic strip card)